# 2.4 – Working in Restricted Ceiling Spaces

Technical Guidance Series – VEU Ceiling Insulation activity

This is part of a series developed with WorkSafe to help installers in our program to work safely while installing insulation.

Use this sheet and others in this series to plan safe series of work while installing insulation.

| VEU Ceiling Insulation – Technical Guidance Series |  |
| --- | --- |
| Working safely at heights |  |
| 1.1 | Working safely at heights during rooftop insulation installations |
| 1.2 | Edge protection – working at height |
| 1.3 | Manual handling of heavy and bulky items |
| 1.4 | Working safely with ladders |
| 1.5 | Safe work practices using elevating work platforms |
| 1.6 | Falls through skylights, fragile roofs, voids and penetrations. |
| Working safely in ceiling spaces |  |
| 2.1 | Working in pairs (best practice) |
| 2.2 | Working near asbestos containing material |
| 2.3 | Working in heated ceiling spaces when installing insulation |
| 2.4 | Working in restricted ceiling spaces (this sheet) |
| 2.5 | Falls through ceiling spaces |
| 2.6 | Fire hazards in ceiling spaces |
| Working safely with electricity |  |
| 3.1 | Safe elements of electrical isolation |
| 3.2 | Safety isolating solar PV systems |

## Restricted ceiling spaces

Entering restricted spaces (such as ceiling cavities) can present several hazards. As part of installing ceiling insulation, it is unavoidable for workers to enter a restricted ceiling space. This means that protective measures need to be put in place.

Hazards can include:

Noise: Noise reverberates inside enclosed spaces, raising the risk of future hearing loss.

Dust: Ceiling spaces are often dusty, raising the risk of respiratory problems unless adequately managed.

Animals: Vermin such as rats, possums, birds or insect infestations can be present in ceiling cavities, with the potential for bites and stings and exposure to animal droppings.

Reduced air flow: Poor ventilation and lack of air movement can reduce the ability of the body to cool down.

Excessive heat: Ceiling spaces are subject to artificial extremes in temperature due to radiant heat from the sun, which can result in internal temperatures of more than 50 degrees Celsius.

Ergonomic: Positioning bodies in certain ways to fit into narrow spaces.

The risks of reduced air flow and excessive heat in ceiling spaces is covered in 2.3 – Working with heat in ceiling spaces. Please refer to that guidance sheet for further information.

#### Managing risks in restricted ceiling spaces

This guidance sheet covers how to identify, assess and manage the risks of working in ceiling spaces.

## Risk: Noise

Summary: Ceiling spaces can become quite noisy due to being enclosed. When equipment is used as part of a ceiling insulation installation, noise may reach hazardous levels, requiring mitigation.

| Risk | Information |
| --- | --- |
| Risk? | Installing ceiling insulation inside ceiling spaces can be quite noisy because ceiling spaces are enclosed spaces where noise can reverberate equipment such as vacuums may be required to remove old insulation. Employees cannot be exposed to noise that exceeds an 8-hour average of 85 db.(A) or a peak of 140 db.(C). |
| Assessing the risk | Consider the likelihood, nature, and severity of potential consequences, and document the hazards, controls, and systems of work within the Job Safety Analysis (JSA) (or equivalent risk assessment process. |
| Methods to reduce risk | Key methods to reduce noise exposure include: Eliminating the source of noise Substitution, like using equipment that creates less noise opening tiles or roof sheets in the roof to allow for noise to dissipate as opposed to reverberating around the ceiling space rotating workers appropriate use of PPE, including ear protection such as earbuds or earmuffs. |

For more information on noise, please see the following WorkSafe materials:

WorkSafe: Compliance Code – Noise: worksafe.vic.gov.au/resources/compliance-code-noise

## Risk: Vermin

Summary: Ceiling spaces often contain vermin, which can pose risks to workers if disturbed. In addition, vermin such as possums or rats may chew through electrical wires, potentially exposing workers to live wires.

| Risk | Information |
| --- | --- |
| Risk? | Vermin commonly inhabit ceiling spaces as they are rarely visited by humans. However, this can pose the following risks to workers. Rats or possums chewing through electrical cabling, leaving live wires present Serious infestations of potentially dangerous pests such as venomous spiders, bats or wasps. |
| Assessing the risk | If vermin are present and pose a hazard, take into account the likelihood, nature and severity of consequences and document the hazards (such as assessing cabling for any damage prior to commencement of work), controls, and systems of work within the JSA (or equivalent risk assessment process). |
| Methods to reduce risk | Key methods to reduce risk from vermin exposure include: appropriate use of PPE, to be worn If wiring is damaged, ensure wiring is repaired by a licensed electrician prior to the commencement of work if infestations are severe or vermin present pose a serious safety risk (such as a wasp nest), stop work and contact an appropriate pest controller to fix the issue before work resumes. |

## Risk: Dust

Summary: Ceiling spaces are often dusty, which when disturbed can reduce air quality. As ceiling spaces are restricted, there is no way for dust to be cleared through airflow, which can result in dusty and unsafe conditions.

| Risk | Information |
| --- | --- |
| Risk | Breathing in excessive amounts of dust is a known health hazard with significant impacts. The severity of these issues depends on: what the dust particles are made of the size of the dust particles how much dust is in the air. In addition, some dusts are known to cause serious lung diseases, such as those composed of respirable crystalline silica. |
| Assessing the risk | If dust is present and poses a hazard, assess the likelihood, nature and severity of potential consequences and implement appropriate control measures. Dust exposure alone does not constitute high-risk construction work. If asbestos-containing material is identified, work must cease and be managed in accordance with asbestos regulatory requirements. For information on when a SWMS is required refer to Worksafe Victoria guidance: worksafe.vic.gov.au/safe-work-method-statements-swms |
| Methods to reduce risk | Key methods to reduce dust exposure should follow the hierarchy of controls, with higher-order controls implemented wherever reasonably practicable before relying on personal protective equipment (PPE). These methods include: Increased ventilation in the work area, where possible to reduce airborne dust concentrations Using local exhaust ventilation or dust extraction systems, where possible, to capture dust at the source. Wearing appropriate, well-maintained and correctly fitted PPE when working in dusty ceiling spaces. This should include: a half face (either P1 or P2) disposable particulate respirator head covering and goggles to avoid eye irritation long sleeved, loose-fitting clothing and gloves. Change of clothes or cleaning facilities. |

## Ongoing risk management

An employer, or person with management and control of a workplace, needs to ensure that measures implemented to control risks associated with working in ceiling spaces are reviewed. This needs to be done when:

the conditions in the roof change or a new hazard is identified

the control measures (such as PPE) are insufficient to manage and control the risk.

Employers need to also review, and if necessary, revise control measures after receiving a request from a Health and Safety Representative (HSR).

Risk management is an ongoing process. Regularly review your control measures to make sure they remain effective and update them when conditions or hazards change.

As a ceiling insulation installer, you will be entering restricted ceiling spaces daily. Awareness of the various risks highlighted in this technical guidance sheet is crucial to ensure ongoing safety while installing insulation.

© The State of Victoria Department of Energy, Environment and Climate Action September 2026.

ISBN 978-1-76176-983-2 (pdf/online/MS word)

Disclaimer

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.

Accessibility

To receive this publication in an alternative format, please contact the Victorian Energy Upgrades team at energy.upgrades@deeca.vic.gov.au. This document is also available on the internet at energy.vic.gov.au/victorian-energy-upgrades.

_______